Luminous BioSolutions

Bacterial biosensor technology for specific detection of naphthenic acids.

The problem

Oil sands mining in the Athabasca region generates large volumes of oil sands process-affected water (OSPW) that is stored long-term in tailings ponds. OSPW contains numerous organic compounds that are naturally present in the oil sands. These include naphthenic acids (NA), a complex family of acyclic and cyclic carboxylic acids, which are toxic to wildlife and humans at the concentrations present in the tailings ponds. Long-term monitoring of ponds and surrounding freshwaters is necessary to ensure environmental and human health in the region, and to support sustainable bitumen mining.

The opportunity

Analytical chemistry methods exist to monitor the presence of naphthenic acids in OSPW. These methods require sophisticated analytical chemistry instruments to detect and characterize the compounds present in OSPW extracts. We propose an alternative biological detection technology for environmental monitoring, which has several advantages compared to conventional methods including low cost, high throughput, simple, specific and sensitive. Biosensors are ideal screening tools to test large numbers of samples, and to therefore provide greater resolution of NA presence in the environment.

The NA biosensor technology

Biosensors are engineered bacteria that are used as a complementary method to indicate the presence of NA in water. Biosensor technology is decades in development, and has been applied to many pollutants, including an early proof of concept for arsenic detection in groundwater. Water samples can be tested directly and after concentration of naphthenic acids. A US and Canadian patent is pending that describes multiple biosensors that can each detect the presence of specific naphthenic acid compounds that are present within the complex OSPW mixes (patent pending). The bacterial chassis strain was isolated from tailings ponds, as it has been naturally selected for survival in that environment and can degrade naphthenic acids. Genomics was used to identify specific genes that are induced after sensing the presence of naphthenic acids in the cell, which are either nutrient uptake and remediation genes, or genes required to efflux the antimicrobial naphthenic acid compounds. Upon rapid detection of naphthenic acids, the chassis strain produces bioluminescence (light) in proportion to the amount of NA in a sample, thereby acting as a semi-quantitative indicator of the presence or absence of NA. When all three NA-detecting biosensors are positive for NA detection, there is a high confidence of NA presence, which has been confirmed using mass spectrometry methods The development of NA biosensors was supported by funding from an NSERC Discovery Grant and Canadian Natural Resources.

Bioremediation of naphthenic acids. In addition to serving as a rapid and simple tool to detect naphthenic acids in water, the biosensors also permit the identification of bacterial strains that can degrade NA. Our current phase of research is directed towards screening hundreds of bacterial isolates recovered from oil sand process-affected water to identify NA degraders, and then to identify a defined, minimal bacterial community with enhanced NA degradation capacities. We envision a bioaugmentation strategy to employ in constructed wetlands water treatment systems, where bacterial cultures comprised of native OPSW bacteria are grown in scale, and added to the wetland, to enhance NA treatments. Currently, the degradation of NA in constructed wetlands relies on low bacterial numbers that are present in OSPW, in addition to plants, and other filtration processes of a wetland. We are currently participating in a large collaborative project between academia and Imperial Oil, where we have the opportunity to field test the NA biosensors with water treated in the constructed wetlands, and to compare orbitrap mass spectrometry analysis.

The platform

Our company is prepared for testing water, sediment or soil samples that are collected on site by the client, and sent to a lab for biosensor testing. This technology is a suitable high throughput approach to screening large numbers of water samples. We can confirm NA detection from a small percentage of the samples, using certified mass spectrometry methods. We are also developing a data analysis and visualization portal that will facilitate the analysis of NA monitoring data over large areas, and across time. INSERT SECTION ABOUT PLATFORM, MENTION AI??

Summary